FESAC TAP ST-Q5

Authors: Rob La Haye, Stan Kaye

Which  scientific and technical questions are unique to the ST?

- Start-up and sustainment without or with only a modest inductive flux-swing coil

- Current dependence on L-H threshold

- Operation in low , high  regime

- Strong BT, weak Ip scaling of energy confinement

- Routine neoclassical energy transport of thermal ions during H-mode

- Large (up to MHZ) ExB shearing rate, and its role in suppression of electron-scale transport-inducing turbulence

- Electron transport losses dominant; role of electromagnetic and electron-scale electrostatic turbulence

- Effect of 90% trapped electrons on CTEM, GAE mode stability

- Large value of I/aB and shaping factor Iq/aB, leading to routine operation at

kink stable high-N

- Possible monotonic q profiles with hollow current

- Curvature stabilization of neoclassical tearing modes

- Routine existence of super-Alfvénic eigenmodes

- RF physics with High Harmonic Heating and Current Drive

- Effect of 90% trapped particles in collisionless outboard SOL

Which S&T questions are low-A extensions of well-known or well-studied conventional tokamak physics?

- RWM, NTM physics and control

- ELM control with RMP, Li evaporation

- Momentum confinement at low B, high edge flows

- NTV, rotation braking in low-to-moderate regime

- Energy confinement and transport: parametric scaling and physics of transport

- Energetic particle confinement, generation of Alfvén eigenmodes, and effect on driven current profile

- Hydrogenic retention/transport

- Heat flux mitigation in compact systems

- Overall, extended parameter space provides high leverage in testing and validating theory

Where does research at low-A help understanding the physics of higher-A tokamaks?

- Trapped particle effects on electrical conductivity, energy transport, AE destabilization, SOL dynamics

- Physics which determining the L-H transition and the H-mode pedestal width

- Edge (ELM) stability

- Physics of neoclassical ion transport, anomalous electron transport

- Physics of AEs with larger gaps and reduced continuum damping; multi-mode coupling

- Kink stabilization by plasma rotation and a resistive wall

- Threshold island width scaling for NTMs

- Rotation braking, NTV physics in low-to-moderate regime

- RF coupling, propagation and Parametric Decay Instability

What are the unique scientific features supplied by the ST that are broadly relevant?

- Non-inductive ramp-up and sustainment

- Dominant, anomalous electron transport

- Role of very high fraction of trapped particles in determining core confinement and SOL dynamics

- Role of high level of ExB shear in suppression of ion- and electron-scale turbulence

- Large value of I/aB and shaping factor Iq/aB lead to high, no-wall kink stable T, and allows operation at very high-N

What issues will ITER answer that apply to the ST?

- Non-inductive ramp-up and sustainment

- Physics of electron transport in electron-heated regimes

- Physics of super-Alfvénic, energetic ’s

- RF coupling physics and Parametric Decay Instability